| Registration No.: | N | Start |  | Aircraft Start Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Start Total Cycles: |  |

Reference:  BHT 407 MM-1 Chapter 5

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - GENERAL |  |  |
| - Logbook review for recorded discrepancies. Correct discrepancies on installed equipment. |  |  |
| - Ensure all applicable and required Special Inspections, Alert Service Bulletins (ASB’s), Engine Service Bulletins, and Airworthiness Directives (AD’s) are accomplished. |  |  |
| - Replace all life limited components that have completed published operating limits. | Bell 407 MP |  |
| - Overhaul all components that have completed published operating limits. | Bell 407 MP |  |
| - FUEL SYSTEM |  |  |
| - Remove the fuel transfer/fuel boost pump assemblies. | Chapter 28 |  |
| - Look into the fuel cells. Examine them for unwanted materiel, water contamination and fungus growth. |  |  |
| - Disassemble, examine and functionally test the fuel transfer fuel boost pump. |  |  |
| - Check thermal relief valves for condition. |  |  |
| - Do a calibration of the fuel quantity system. | Chapter 95 |  |
| - Look into the auxiliary fuel tank if installed. Examine for unwanted materiel, water contamination and fungus growth. | BHT-407-MMS-6 Chapter 99 |  |
| - CONTROLS |  |  |
| - Inspect the cyclic spring link for condition. | Chapter 67 |  |
| - FLIGHT CONTROL BOLTS/NUTS |  |  |
|  | Chapter 67 |  |
| - Examine the bolts and the nut of the flight controls for condition. |  |  |
| - Examine the bypass spool valve of the servo actuator for correct movement. | Chapter 29 |  |
| - POST INSPECTION |  |  |
| - Complete a ground run at 100% NR to check for leaks and confirm system operation. | BHT-407-FM-1/2 |  |
| - Replace or close all applicable trim panels, covers, access and inspection doors/covers as necessary. |  |  |
| - Perform Functional Check Flight if required per Bell 407 Maintenance Manual or RFM as appropriate. |  |  |
| - Make an entry in the appropriate logbook (refer to the following page for wording of approval for return to service). |  |  |

Upon satisfactory completion of each inspection item, the mechanic shall initial the pertinent space as provided.  All mechanics shall sign and initial, in the space provided, at the end of the form.

At the completion of each inspection form, an A&P mechanic shall check the entire form for completion and then enter his/her signature, certificate type and number, and date in the space provided at the end of the form.  The same individual is also responsible for making the logbook entry shown at the conclusion of this inspection.

I certify that I have inspected and/or completed all the items initialed by me in accordance with Maritime Helicopters’ Bell 407 Approved Aircraft Inspection Program and found them to be in an airworthy condition except as may be noted in the Aircraft Logbook.

| INITIALS | SIGNATURE | CERTIFICATE TYPE & NUMBER |
| --- | --- | --- |

I have reviewed this inspection form to verify that all items have been completed and have made an entry in the Aircraft Log Book.

I certify that this aircraft has been inspected in accordance with Maritime Helicopters’ Bell 407 AAIP – A 2400 – 24 Month Inspection and was determined to be in an airworthy condition.

| Registration No.: | N | End |  | Aircraft Ending Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Ending Total Cycles: |  |

| Signature: |  |  | Certificate Type & No: |  |
| --- | --- | --- | --- | --- |