| Registration No.: | N | Start |  | Aircraft Start Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Start Total Cycles: |  |

Reference:  AW119MKII MPM Chapter 05, 05-70

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - GENERAL |  |  |
| - Logbook review for recorded discrepancies. Correct discrepancies on installed equipment. |  |  |
| - Ensure all applicable and required Special Inspections, Alert Service Bulletins (ASB’s), Engine Service Bulletins, and Airworthiness Directives (AD’s) are accomplished. |  |  |
| - Replace all life limited components that have completed published operating limits. | AW119MKII MPM |  |
| - Overhaul all components that have completed published operating limits. | AW119MKII MPM |  |
| - PLACARDS AND MARKINGS |  |  |
| - Examine the placards, decals, and markings. Make sure you can read them, they are applied correctly, and they are in agreement with the applicable configuration of your helicopter. | Chp 11 |  |
| - ROTOR DRIVE |  |  |
| - MAIN TRANSMISSION STRUCTURAL FWD AND AFT LOWER FITTINGS |  |  |
| - Inspect main transmission structural fwd and aft lower fittings for corrosion, cracks and proper condition of joints. | 63-31-10 |  |
| - Lubricate main rotor servo actuators (3 places) |  |  |
| - Lubricate main rotor swashplate (2 places) with C594 or C009 Do not mix! | 12-20-5 |  |
| Note: Select which lubrication; ____ 1st ____ 2nd ____ 3rd ____ 4th |  |  |
| Type of Grease ______________________ |  |  |
| - In regards to the above, every 4th lubrication: |  |  |
| - On upper swashplate bellow cut lower safety wire (ref. Fig. 62-45, detail A1), or remove the clamp (whatever installed), and lift the bellow to expose bellow ring support. Lift ring support bellow and remove all visible grease extruding from duplex bearing seat. | 12-20-5 |  |
| CAUTION DO NOT MIX GREASE C594 WITH C009. MAKE AN APPROPRIATE ENTRY IN HELICOPTER LOG BOOK TO INDICATE THE GREASE USED TO PREVENT SUBSEQUENT SERVICING WITH A DIFFERENT BRAND |  |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - FLIGHT CONTROLS |  |  |
| - Service servo hydraulic system accumulators (2 places) with nitrogen | 12-20-13 |  |
| - POST INSPECTION |  |  |
| - Replace or close all applicable trim panels, covers, access and inspection doors/covers as necessary. |  |  |
| - Complete a ground run at 100% NR to check for leaks and confirm system operation. |  |  |
| - Perform Functional Check Flight if required per AW119MKII MPM Maintenance Manual or RFM as appropriate. |  |  |
| - Make an entry in the appropriate logbook (refer to the following page for wording of approval for return to service). |  |  |

Upon satisfactory completion of each inspection item, the mechanic shall initial the pertinent space as provided.  All mechanics shall sign and initial, in the space provided, at the end of the form.

At the completion of each inspection form, an A&P mechanic shall check the entire form for completion and then enter his/her signature, certificate type and number, and date in the space provided at the end of the form.  The same individual is also responsible for making the logbook entry shown at the conclusion of this inspection.

I certify that I have inspected and/or completed all the items initialed by me in accordance with Maritime Helicopters’ AW119MKII Approved Aircraft Inspection Program and found them to be in an airworthy condition except as may be noted in the Aircraft Logbook.

| INITIALS | SIGNATURE | CERTIFICATE TYPE & NUMBER |
| --- | --- | --- |

I have reviewed this inspection form to verify that all items have been completed and have made an entry in the Aircraft Log Book.

I certify that this aircraft has been inspected in accordance with Maritime Helicopters’ AW119MKII MPM AAIP – A 0250RT – 200 Hour / 12 Month Inspection and was determined to be in an airworthy condition.

| Registration No.: | N | End |  | Aircraft Ending Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Ending Total Cycles: |  |

| Signature: |  |  | Certificate Type & No: |  |
| --- | --- | --- | --- | --- |