| Component | Action | Interval |
| --- | --- | --- |
| Oil Quantity | Do a visual inspection of the oil level. Fill as necessary (Ref. 72-00-00, ENGINE - SERVICING). | Before engine start (or Maximum interval of two flight hours) from engine last operation above 15000 feet + below 60% Ng |
|  |  | Daily - for all other operating conditions |
| Oil Filter Bypass Indicator | Check for oil filter bypass pop-out indicator (Ref. Rotorcraft Flight Manual). | Daily |
| Mechanical Mode Check | Perform mechanical mode check (Ref. Rotorcraft Flight Manual). | Daily |
| Ground Idle and Flight Idle Functional Checks | For Pre-SB39014/Post-SB39014 Fuel Control Units with Pre-SB39027 fuel pumps perform ground idle and flight idle speed functional checks (Ref. 71-00-00, ADJUSTMENT/TEST). | Daily |
| Fuel Control Unit | For Pre-SB39014 Fuel Control Units perform FCU override cover checks per P&WC Service Bulletin A39013. | Per instructions on P&WC Service Bulletin A39013 |
| Manual Override System Static Check | Do a manual override system static check (Ref. 71-00-00, POWER PLANT - ADJUSTMENT/TEST). | 50 Hours - B 0050 |
| Control System Stability Check | Do a control system stability check (Ref. 71-00-00, POWER PLANT - ADJUSTMENT/TEST). | 50 Hours - B 0050 |
| Mechanical Mode Check | Do a mechanical mode check (Ref. 71-00-00, POWER PLANT - ADJUSTMENT/TEST). | 50 Hours - B 0050 |
| Ground Idle Check | For Post-SB39014 fuel control units with Post-SB39027 fuel pump perform ground idle functional check (Ref. 71-00-00, ADJUSTMENT/TEST) | 100 hours - B 0100 |
| EEC Engine Trim Check | Do an EEC engine trim (Ref. 71-00-00, ADJUSTMENT/TEST). | 100 hours - B 0100 |
| Oil to Fuel Heater | Immediately after shutdown: Check the temperature of the heater at the heater fuel outlet or the low pressure (LP) filter housing. If the temperature is more than 140°F (60°C) replace the oil to fuel heater. | 400 hours |
|  | Immediately after shutdown apply a temperature recorder to the LP filter housing. Replace the oil to fuel heater if the center of the recorder becomes black. |  |
|  | NOTE: Use a Temp-Plate Temperature Recorder (PWC05-329) to see if the filter housing becomes hotter than 140°F (60°C). |  |
| Gas Generator Case | Cracks, distortion, corrosion and evidence of overheating. | 400 hours or 12 months whichever comes first |
| Compressor Bleed Valve Pre-SB39024 | Perform in-situ inspection/check of compressor bleed valve (Ref. 72-00-01). | 300 hours |
| Turbine Exhaust Duct | Cracks or distortion. | 600 hours |
| P3 Air Filter Element | Clean and inspect (Ref. 72-00-24, P3 AIR FILTER - MAINTENANCE PRACTICES). | 100 hours - B 0100 |
| P3 Air Filter Drain Valve | Clean and inspect (Ref. 72-00-24, P3 AIR FILTER - MAINTENANCE PRACTICES). | 100 hours - B 0100 |
|  | NOTE: Adjust frequency based on environment. |  |
| Fireseals | Cracks and security of brackets. | 800 hours |
| Tubing | Check resistance of lead and heating element of heated pneumatic tubes using a suitable ohmmeter or resistance bridge (Ref. 72-00-08). | 800 hours or 12 months whichever comes first |
|  | Check for security of all accessible connections, clamps and brackets, evidence of wear, chafing, cracks and corrosion and evidence of fuel or oil leaks. Examine insulation on pneumatic tubes for cuts in outer rubber sheaths. |  |
|  | NOTE: Damage limits and repair instructions for tubing (Ref. 72-00-08, PNEUMATIC LINES - MAINTENANCE PRACTICES). |  |
| Oil Filter Element | Replace filter element. | 600 hours |
| Compressor Inlet Area | At the required frequency and whenever condition of inlet screen warrants its removal, check compressor inlet area for dirt deposits and erosion and check first-stage blades and vanes (Ref. 05-50-00). | 300 hours or 12 months, whichever comes first (Ref. NOTE) |
|  | NOTE: Engines operating in a highly erosive environment and/or conditions where there is a high potential for blade damage require more frequent inspections (Ref. 05-50-00) |  |
| Chip Detectors | Accessory Gearbox - one detector Reduction Gearbox - one detector Check for metallic pick-up and clean using lint-free cloth. Using suitable ohmmeter check for continuity of electrical circuit when the two poles are shorted together. Reject component if no continuity. Loose pins and broken potting compound are also cause for rejection. Perform lift test to check magnetism (Ref. 72-00-19). | 600 hours |
| Ignition System | Check security, cleanliness and condition. Functional check (Ref. 72-00-02). Inspect ignition exciter connectors for corrosion. | 800 hours |
| Igniter Plugs | Inspect for wear and erosion. | 800 hours |
| Fuel Pump | Check for security and fuel leaks. | 800 hours |
|  | If fuel pump gear-set and coupling operating time since new (TSN) is LESS THAN 3500 hours: |  |
|  | (a) Check fuel pump coupling in-situ for fretting and corrosion (Ref. 72-00-22, FUEL PUMP - MAINTENANCE PRACTICES). | Every 600 hours |
|  | (b) Remove fuel pump and inspect the drive coupling and cover accessory gearbox side for signs of reddish-brown (iron oxide) stains. If stains are observed, return the fuel pump to an approved overhaul facility (72-00-22, FUEL PUMP - MAINTENANCE PRACTICES). | Every 1500 hours |
|  | If fuel pump gear-set or coupling operating time since new (TSN) is MORE THAN 3500 hours OR is UNKNOWN OR fuel pump was overhauled before 2007: | ACFT at 1500 hr will standby on this. |
|  | (a) Remove fuel pump and inspect the drive coupling and cover accessory gearbox side for signs of reddish-brown (iron oxide) stains. If stains are observed, return the fuel pump to an approved overhaul facility (72-00-22, FUEL PUMP - MAINTENANCE PRACTICES). | 300 hours |
|  | NOTE: As of January 2010, the fuel pump gear-set and drive coupling replacement is required at every pump overhaul. |  |
| Fuel Filter | Install new low pressure fuel filter (Ref. 72-00-23). | 600 hours |
| Fuel Filter Post-SB39027 | Install new high pressure pump fuel filter (Ref. 72-00-22). | 600 hours |
| Fuel Nozzle Assemblies | Functional Check (Ref. 72-00-04). For improved hot-section durability, inspect and clean fuel nozzle assemblies. | 600 hours |
|  | NOTE: 1. As an alternative to the removal for inspection and functional check, a 300 hour in-situ cleaning procedure is offered (Ref. 72-00-00, ENGINE-CLEANING). |  |
|  | If in-situ cleaning is used the fuel nozzle assemblies shall be removed for inspection and functional check (Ref. 72-00-04) to coincide with airframe maintenance schedule. | 1200 hours |
| Compressor Bleed Valve Pre-SB39024 | Disassemble and inspect compressor bleed valve (Ref. 72-00-01). | 600 hours |
| Compressor Bleed Valve Post-SB39024 | Remove the compressor bleed valve and return it to an authorized accessory overhaul shop for inspection and test (Ref. 72-00-01). | 1500 hours |
| FCU Manual Override Check | Perform functional check (Ref. Rotorcraft Maintenance Manual). | 600 hours |
| Electrical Wiring Harness | Do a visual check on the wiring harness for evidence of chafing, cracks, corrosion and wear. Verify connector security (Ref. 72-00-26, ELECTRICAL WIRING HARNESS - MAINTENANCE PRACTICES). | 800 hours or 12 months whichever comes first |
| EEC Connectors Check | Do a visual check of the J2 connector and pins for evidence of corrosion and wear. Clean and use of contact enhancer (PWC05-256) if required (Ref. 70-00-00, STANDARD PRACTICES - CLEANING). | 800 hours or 12 months whichever comes first |
| T5 System | Carry out functional check (Ref. 05-50-00) at major inspection. Check security of all accessible connections, clamps and brackets and for evidence of wear, chafing, cracks and corrosion. | 600 hours |
| Compressor Turbine Blades | Send compressor turbine disk assembly to an approved overhaul facility for blade replacement (Ref. Creep Usage Counting, Para. 4.). | 1500 ± 25 hours |
| Power Turbine Blades | Send power turbine disk assembly to an approved overhaul facility for blade replacement (Ref. Creep Usage Counting, Para. 4.). | 1500 ± 25 hours |
| No. 10 Bearing | Do an inspection of No. 10 bearing for axial displacement (Ref. 72-10-09, TORQUEMETER - MAINTENANCE PRACTICES). | 1500 ± 25 hours |