Summary of Changes

Global

Page number system was changed to current CAMP/AAIP system for standardization, ease of revisions.

Powerplant Section removed.  These items were already in Viking airframe inspection making most duplicate.  Migrated all that were not into the A 0125 and B 0250 as appropriate to manufacturers interval requirements.

R A007 & R E007 now combined into single 7-day Routine Insp, R 0007

Table of Contents

Title Page – Rev 2

Pg iii – Section 7 now Aircraft Log Page. Section 8 Inspections.  Added ATTACHMENTS. Page iv was not needed for continuation, now Manual Control

Pg iv – Manual Control updated to Rev 2

Log of Effective Pages

Pg vi thru viii – Updated effected pages to Rev 2 12/30/21

Pg ix – Updated, removed POWERPLACE INSPECTIONS

Preamble

Pg 1-1 – Removed “Test” from some items; redundant. Section is already labeled INSPECTIONS/TESTS

Pg 4-2 – Corrected Section first letters from “B” to “C” for C inspections. Removed POWERPLANT INSPECTIONS

Pg 7-1 – Added Aircraft Log Page

Pg 8-1 – Replaces Page 7-1 AIRFRAME INSPECTION with just 8 INSPECTIONS

NOTE; All pagination changes from end of preamble until ATTACHMENTS at end of document.  Now each each inspection has own numbering system, so will only address changes in the body of each inspection sheet.

R 0007 (A 0007 and E 0007 inspections combined & renamed to R 0007)

Page 1

Removed “7.1 ROUTIINE A INSPECTIONS” from title

3.1 - Changed Appendix C to B

4.2 - No escape hatch doors in this aircaft, removed

Page 2

3.2 – Removed Note from 5.5, not applicable

Page 3 & 4

Wing inspections from old R E007. POST INSPECTION RII inspections blocks now colored in; not required

Page 5

Updated new inspection name in sign off

A 0125

Page 1

Aded PT6A MM to Reference:

1.3, 1.4  –  Added PTA6 MM to REF column as inspections from old P 0100 have been incorporated

3.1 – Changed Appendix C to B

Added 32 - LANDING GEAR NOSE (company requuirement)

5.2, 6.2 –  Removed AD note on Overspeed Gov; no applicable

Page 2

6.4 – (Prop RH & LH) Added blending nicks to 6.4.

5.5, 6.5 - No gas tach or prop gen, changed to Prop tach gen,

5.6, 6.6- No prop synch, replaced with shaft seal

8.1,2 & 9.1,2 – Incorporated old Powerplant P 0100 inspections verbiag

10.1, – Incorporated old Powerplant P 0100 inspections verbiag

72 – ENGINE – RH Incorporated old Powerplant P 0100 inspections verbiag

Page 3

10.4,5 – Incorporated old Powerplant P 0100 inspections

11.1-5 – Incorporated old Powerplant P 0100 inspections

12.1-7 – Incorporated old Powerplant P 0100 inspections

Page 4

12.8 thru 13.9 – Incorporated old Powerplant P 0100 inspections, High Pressure filter element removed, same as 12.3, P3 insp’s moved to CALM

Page 5

16.2, 17.2 – Removed Prop gen, already previously addressed in 5.5 & 6.5 Pg 2

18.2, 19.2 – P 0100 items added

Page 5

79 – OIL – LH Mag chip tests are required every 100 hr, will be tracked on CALM as there is no 100 hr CAMP interval

Page 6

79 – OIL – RH Same with Mag test above.

80 – STARTING LH & RH – Our aircraft does not have glow plugs, replaced with ignition current regulator

Page 7

No high pressure fuel filter in aircraft.

Filled in RII blocks. Log entry does not need an RII, or checking oil levels

A 0250

Page 1

Aded PT6A MM to Reference:

1.3,4 – Added PTA6 MM (because powerplant inspections were merged in),

21 – AIRCONDITIONING removed, not installed

Page 2

12.1, 13.1 - No 74-micron elements on this aircraft,

12.4, 13.4 - No wing tanks

Page 4

Colored in Nose wheel assy RII block

Page 6

61 – PROPELLERS LH & RH Removed AD N/A

23.1, 24.1 Nick, cracks and blending already done every 125 hr in A 0125 6.4 and 7.4

72 – ENGINE LH & RH Merged engine inspections from old P 0400

Page 7

80 – STARTING LH & RH has merged in engine inspections from old P 0400

Page 8

Fill in RII block on 26.6

Page 9

Fill in RII block on 27.4 and 27.5

C 0500

Page 1

Added Viking MM to Reference:

3.1 – Changed to Appx B

21 – AIRCONDITIONING Removed, not installed

Page 2

6.7 - No VOR duel selector relay installed

6.10.3 - Shaded in Function Test RII block, clarified “ground” test

Page 3

No HF Radio or inverters installed (-300 aircraft), SOO 6371 not installed

7.16 SOO 6371 not installed, inserted inspection of cables

Page 4

8.1.6 - No “Portable ELT”, just used insp to look at ELT mounting

8.3 - No baggage restraints installed

Page 5

9.12 - No stall warning light

Page 6

28-FUEL – 10.3, shaded in RII. No wing fuel tanks installed (10.4&5)

29 - HYD PWR - Shadded in RII block for hyd filter

Page 7

30 -ICE AND RAIN No windshielf washer system (12.8&9), or engine air intake anti-cing boots (12.10&11)

13.1.2 – Shaded in RII block

Page 9

15.15 -  Removed, only 1 TCAS antenna

15.16 – Removed, no lower TCAS antenna

15.18 – Removed, only 1 ADF antenna

36 – PNEUMATIC no bleed air strainers (16.30)

Page 10

18.3 - No airstair door, aftermarket IKHAHA FWD door installed only

Page 11

18.6 – Removed, no cabin roof hatch installed

Page 12

61 – PROPELLERS – LH & RH No autofeather limit switches or propeller synchronizer systems

Page 14

Removed 73 – ENGINE FUEL & CONT LH & RH; applicable to the  -27 engine only.

76-ENG CONTROLS Removed Note, SB not applicable.

Page 15

80 – STARTING LH & RH No spark ignitors on our engines

Page 16

36.1.5.4 Removed, no Prop syncrhonizers

36.1.8.2 Removed, no instrument pneumatic system strainers

36.1.8.5 Removed, no HP fuel filters

Page 17

Filled in RII blocks, not requred

C 1000RT

Page 1

3.1 – Changed to Appx B

Page 2

RMI & Annuciator not installed on our aircraft

Page 4

Pilot’s/CoPilots “mating” surfaces are from mating tray to airframe

Page 6

10.1.7 Shaded in RII block

Page 11

Removed everything from “Inspect CVR” to next segment, already in 14.25 thru 14.28

Page 13

Removed everything from 11.10 until end of segment.  Wing tanks not installed

Page 21 & 22

A few RII blocks got shaded in for items that did not really need an RII

Page 38

Segment 73 – ENGINE FUEL AND CONT LH & RH removed, P3 now tracked on CALM

OPERATIONAL CHECKS – Engine driven air pumps not intalled on this aircraft (36.2&3)

C 1500RT

Page 1

3.1 – Changed to Appx B

Page 2

5.13 – Removed, DeVore bleed air ducts not installed

6.1 – VHF/FM Removed, not installed

7.2 Volt meter rempoved, not installed.

7.3 Load meter removed, not installed

Page 3

11.5 & 6 removed, no wing tanks

Page 5

16.1 This type of light (brushes) not installed, changed to just inspect light

16.2 Removed, no lower beacon light

Page 6

17.1 - Already listed under 6.2 Communications

18.1.4 – Removed, no eng press ind installed. Now ID’s thru CAS system

18.1.5 - Removed, no eng driven air pumps installed

18.1.6 - Removed, no eng air pump system installed, workes with bleed air valve in cockpit

18.1.12 - Removed, no engine pressure indicators installed. Done thru CAS system

18.1.13 - Removed, no engine driven air pumps

18.1.14 - Removed, no eng air pump system installed, workes with bleed air valve in cockpit

18.1.15 - Removed, no Emergency instrument vacuum installed on 400 series

18.1.16 - Removed, no pressure and suction indicator installed on 400 series

Page 7

Shaded in multiple RII boxes that were inadvertently left open

Page 8

53 – FUSELAGE Shaded in RII boxes in that were inadvertently left open

Page 12

Removed strob light pwr supply, already covered in 33 - LIGHTS

Page 13

Removed strob light pwr supply, already covered in 33 - LIGHTS

Page 14

27.1.5 Shaded in RII

Page 16

28.6 – No collector tanks, just changed to insp fuel drain lines.

28-10 – Removed, nacelle waste fuel return line and connections.  No collector tanks on our aircraft

28-11 – Removed, not installed on our aircraft

Page 19

Shaded in RII for Function Check flight and Log Book Entry.

C 1700NT

Page 1

3.1 – Changed to Appx B

5.1.4 – Added tension requirements

Page 2

5.2.4 – Added tension requirements

5.3.4 – Added tension requirements

Page 2

Removed 23 – COMMUNICATIONS Mod not installed

Page 6

Removed 34 NAVIGATION Tracked on CALM

Page 8

Shaded in RII for Function Check flight and Log Book Entry.

C 3000NT

Page 1

3.1 – Changed to Appx B

Page 3

36 – PNEUMATIC Pneumatic, the system described in this section is for the earlier model Twin

Otters, 100, 200, and 300 series. Replacing with components that are currently installed on the 400 series aircraft.

15-1 – Removed, automatic water drain is not installed on the aircraft.

15.2 – Modified inspection for installed system

Page 4

52 – DOORS removed, both items not installed on our aircraft

Page 5

17.12 – Removed, no cabin roof hatch installed

17.13 – Removed, repeat of 17.2 in same section

Page 8

61 – L/H PROPELLERS removed Prop sych & Beta range system, not installed. Now auto feather system relay box

61 – R/H PROPELLERS removed Prop sych & Beta range system, not installed. Now auto feather system relay box

Page 10

Shaded in RII for Function Check flight and Log Book Entry.

CT 6000NT

Page 1 – Removed 7.4 COMPONENT TESTS from heading

F 3000NT

Page 1 – Removed 7.5 FUNCTIONAL CHECKS from heading