| Registration No.: | N | Start |  | Aircraft Start Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Engine Start Total Time: |  |
| Engine S/N: |  |  |  | Engine Start Total Cycles: |  |

Reference:  BHT 412 MM-9, Pratt & Whitney PT6T MM

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - GENERAL |  |  |
| - Logbook review for recorded discrepancies. Correct discrepancies on installed equipment. |  |  |
| - Ensure all applicable and required Special Inspections, Alert Service Bulletins (ASB’s), Engine Service Bulletins, and Airworthiness Directives (AD’s) are accomplished. |  |  |
| - Replace all life limited components that have completed published operating limits. | Pratt & Whitney |  |
|  | PT6T MM |  |
| - Overhaul all components that have completed published operating limits. | Pratt & Whitney |  |
|  | PT6T MM |  |
| - PLACARDS AND MARKINGS |  |  |
| - Examine the placards, decals, and markings in engine area. Make sure you can read them, they are applied correctly, and they are in agreement with the applicable configuration of your helicopter. | BHT 412 MM Chapter 11 |  |
| - CORROSION CONTROL |  |  |
| - Perform corrosion inspection in accordance with the Corrosion Guide on any area’s access during this CAMP inspection. | CSSD-PSE-87-001 |  |
| - GENERAL |  |  |
| - Remove all panels, fairings, and cowlings required to satisfactorily carry out the inspection. |  |  |
| - COMPONENTS |  |  |
| - Fuel Surge Accumulator |  |  |
| EFFECTIVITY: Post-SB5398 |  |  |
| - Visually inspect. Replace accumulator if pop-out indicator is extended. | 73-10-03 |  |
| - Fireseals |  |  |
| - Cracks and security of brackets. |  |  |
| - Bleed Air Case Assembly |  |  |
| EFFECTIVITY: Post-SB5445 |  |  |
| - Check the orientation of the bleed air case assembly. | 72-20-00 |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - Inlet Screen |  |  |
| - Cleanliness and condition of mesh. |  |  |
| - Perform visual inspection of the compressor air inlet screen for broken and or missing wire mesh segments and signs of brittleness. |  |  |
| - Check integrity of the mesh by manipulating and flexing the screen. Any deviation is a cause for replacement. |  |  |
| - If the inlet screen is removed, do an inspection of first-stage compressor blades. | 72-00-00 Para 9.B ENGINE |  |
|  | INSP/CHECK |  |
| - Do a general visual inspection (GVI) of wash-ring assembly and safety wire for security and obvious damage. If any deviation is found during GVI, do an inspection of first-stage compressor blades per Para. 9.B. | 72-00-00 ENGINE INSP/CHECK |  |
| - Oil Filter Element (10 micron) |  |  |
| - Clean (Electrosonic or Ultrasonic Bath Method). | 79-20-02 |  |
| - Inspect filter element. |  |  |
| - Chip Detectors |  |  |
| - Accessory Gearbox - 2 detectors; |  |  |
| - Clean with lint-free cloth. |  |  |
| - Measure the minimum weight lift capacity. |  |  |
| - Use a suitable ohmmeter to check the continuity of the electrical circuit when the two poles are shorted together. |  |  |
| NOTE: Reject the component if there is no continuity. Loose pins and broken potting compound are also causes for rejection. |  |  |
| - Pre-SB5354: |  |  |
| - Inspect colored stripes painted on connector and housing. |  |  |
| NOTE: If stripes are displaced, reject chip detector. If no stripes are apparent, paint three equally spaced colored stripes on connector and housing using suitable paint. Connector threads must be free of paint. Refer to Chapter 79-30-01 for further inspection details. |  |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - Reduction Gearbox - 3 detectors; |  |  |
| - Clean with lint-free cloth. |  |  |
| - Measure the minimum weight lift capacity. |  |  |
| - Use a suitable ohmmeter to check the continuity of the electrical circuit when the two poles are shorted together. |  |  |
| NOTE: Reject the component if there is no continuity. Loose pins and broken potting compound are also causes for rejection. |  |  |
| - Pre-SB5354; |  |  |
| - Inspect colored stripes painted on connector and housing. |  |  |
| NOTE: If stripes are displaced, reject chip detector. If no stripes are apparent, paint three equally spaced colored stripes on connector and housing using suitable paint. Connector threads must be free of paint. Refer to Chapter 79-30-01 for further inspection details. |  |  |
| - POST INSPECTION |  |  |
| - Replace or close all applicable trim panels, fairings, covers, access and inspection doors/covers as necessary. |  |  |
| - Complete a ground run at 100% NR to check for leaks and confirm system operation. | BHT-412-FM-1 |  |
| - Perform Functional Check Flight if required per Bell 412 Maintenance Manual or RFM as appropriate. |  |  |
| - Make an entry in the appropriate logbook (refer to the following page for wording of approval for return to service). |  |  |
| - Confirm all required RII have been completed and signed off. |  |  |

Upon satisfactory completion of each inspection item, the mechanic shall initial the pertinent space as provided.  All mechanics shall sign and initial, in the space provided, at the end of the form.

At the completion of each inspection form, an A&P mechanic shall check the entire form for completion and then enter his/her signature, certificate type and number, and date in the space provided at the end of the form.  The same individual is also responsible for making the logbook entry shown at the conclusion of this inspection.

I certify that I have inspected and/or completed all the items initialed by me in accordance with Maritime Helicopters’ Bell 412 Continuous Airworthiness Maintenance Program and found them to be in an airworthy condition except as may be noted in the Aircraft Logbook.

| INITIALS | SIGNATURE | CERTIFICATE TYPE & NUMBER |
| --- | --- | --- |

I have reviewed this inspection form to verify that all items have been completed and have made an entry in the Aircraft Log Book.

I certify that this aircraft has been inspected in accordance with Maritime Helicopters’ Bell 412 CAMP – B 0150 – 150 Hour Engine Inspection and was determined to be in an airworthy condition.

| Registration No.: | N | Start |  | Aircraft Start Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Engine Start Total Time: |  |
| Engine S/N: |  |  |  | Engine Start Total Cycles: |  |

| Signature: |  |  | Certificate Type & No: |  |
| --- | --- | --- | --- | --- |