Altimeter 24 Month Checklist

| ITEM No. | Altimeter Appendix E (91.411) |  |  |  |  |  | INSPECT DATE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NOTE: Appropriate Manufactures Maintenance Manuals will be used during any inspection of unfamiliar systems on the aircraft. |  |  |  |  |  |  |
| 1 | Ensure the Pitot/Static system is free from entrapped moisture and restrictions. Drain system prior to test if applicable. |  |  |  |  |  |  |
| 2 | Check operation of pitot tube and/or static port heaters if installed |  |  |  |  |  |  |
| 3 | Ensure no alterations or deformations of the airframe surface have been made that would affect the relationship between air pressure in the static pressure system and true ambient static air pressure for any flight condition. |  |  |  |  |  |  |
| 4 | Set the Altimeter on the test set and the aircraft to 29.92 and note indicated altitude. Record indicated Altitude: A) Test Set Altimeter:_________ B) Aircraft Altimeter: L_______ R_______ |  |  |  |  |  |  |
| 5 Scale Error Check NOTE: ITEMS 1-5 WILL BE UTILIZED WHEN PERFORMING A DATA CORRE- SPONDANCE TEST (APPENDIX E, para c) | Bring the test set to each of the following test points Let stabilize for at least one minute but not more than 10. Enter the aircraft altimeter indication and calculate error. This error must not exceed the max tolerance. |  |  |  |  |  |  |
|  | TEST SET ALTITUDE | MAX TOLERANCE | AIRCRAFT INDICATED ALT |  | ERROR |  | NOTE: CASE LEAK AT 18,000 FEET FOR 1 MIN. NOT TO EXCEED 30 FEET CASE LEAK _____Feet |
|  | -1000 | 20 | L | R | L | R |  |
|  | 0 | 20 | L | R | L | R |  |
|  | 500 | 20 | L | R | L | R |  |
|  | 1000 | 20 | L | R | L | R |  |
|  | 1500 | 25 | L | R | L | R |  |
|  | 2000 | 30 | L | R | L | R |  |
|  | 3000 | 30 | L | R | L | R |  |
|  | 4000 | 35 | L | R | L | R |  |
|  | 6000 | 40 | L | R | L | R |  |
|  | 8000 | 60 | L | R | L | R |  |
|  | 10,000 | 80 | L | R | L | R |  |
|  | 12,000 | 90 | L | R | L | R |  |
|  | 14,000 | 100 | L | R | L | R |  |
|  | 16,000 | 110 | L | R | L | R |  |
|  | 18,000 | 120 | L | R | L | R |  |
|  | 20,000 | 130 | L | R | L | R |  |
|  | 22,000 | 140 | L | R | L | R |  |
|  | 25,000 | 155 | L | R | L | R |  |
|  | 30,000 | 180 | L | R | L | R |  |
|  | 35,000 | 205 | L | R | L | R |  |
|  | 40,000 | 230 | L | R | L | R |  |
|  | 45,000 | 255 | L | R | L | R |  |
|  | 50,000 | 280 | L | R | L | R |  |
|  | NOTE: Within 15 minutes of the initial exposure to upper limits of the scale in step 5 perform step 6. |  |  |  |  |  |  |

| ITEM No. | Altimeter Appendix E (91.411) |  |  |  |  |  |  |  | INSPECT DATE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 Hysteresis | From max altitude test point, rapidly decrease attitude at the max rate of descent of the aircraft VSI until within 3000 ft of the first test point (50% of the max altitude tested) then approach the test point at the approximately 3000 ft/min. Stabilize 5 minutes (max 75’ diff from item 5) Aircraft Alt Reads: Left:_______________ Right:______________ |  |  |  |  |  |  |  |  |
|  | Rapidly decresse attitude at approximately 3000 ft/min to the second test point. (40% of max altitude tested) Stabilize one (1) minute: Aircraft Alt Reads: Left:______ Right:_______ (max 75’ diff from item 5) |  |  |  |  |  |  |  |  |
| 7 After Effect | Not more than 5 minutes after completion of Step 6, equalize pressure to ambient pressure. Record altitude. (Max difference is 30’ from item 4, B) Aircraft Alt Reads: Left:________ Right:__________ |  |  |  |  |  |  |  |  |
|  | Increase altitude at 750 feet per minute to each of the altitudes listed below. While still increasing altitude, tap on the panel next to the altimeter and note the amount the needle jumps, enter below |  |  |  |  |  |  |  |  |
|  | Altitude |  | Tolerance |  | Enter change in reading of pointer |  |  |  |  |
|  | 1000 |  | 70 |  | L |  | R |  |  |
|  | 2000 |  | 70 |  | L |  | R |  |  |
|  | 3000 |  | 70 |  | L |  | R |  |  |
|  | 5000 |  | 70 |  | L |  | R |  |  |
|  | 10,000 |  | 80 |  | L |  | R |  |  |
|  | 15,000 |  | 90 |  | L |  | R |  |  |
|  | 20,000 |  | 100 |  | L |  | R |  |  |
|  | 25,000 |  | 120 |  | L |  | R |  |  |
|  | 30,000 |  | 140 |  | L |  | R |  |  |
|  | 35,000 |  | 60 |  | L |  | R |  |  |
|  | 40,000 |  | 180 |  | L |  | R |  |  |
| 9 Baro- Metric Scale Error | A) Begin at 29.92 on the test set and the aircraft: Test Set Altimeter:_________ Aircraft Altimeter: L_______ R_______ |  |  |  |  |  |  |  |  |
|  | B) Turn the Barometric pressure knob to the listed pressures in column i |  |  |  |  |  |  |  |  |
|  | C) Record the actual indicated altitude of the aircrafts altimeter in column iii |  |  |  |  |  |  |  |  |
|  | D) Calculate the difference between the starting altitude recorded in 9, A and the indicated altitude of the required check point. Record actual diff. in column iv. |  |  |  |  |  |  |  |  |
|  | i) Pressure | ii) Alt Diff |  | iii) Ind. Alt |  | iv) Actual Diff (MAX 25) |  |  |  |
|  | 28.10 | -1,727 |  |  |  | L |  | R |  |
|  | 28.50 | -1,340 |  |  |  | L |  | R |  |
|  | 29.00 | -863 |  |  |  | L |  | R |  |
|  | 29.50 | -392 |  |  |  | L |  | R |  |
|  | 29.92 | 0 |  |  |  | L |  | R |  |
|  | 30.50 | +531 |  |  |  | L |  | R |  |
|  | 30.90 | +893 |  |  |  | L |  | R |  |
|  | 30.99 | +074 |  |  |  | L |  | R |  |
| 10 | Altimeters which are air data computer type with associated systems may be tested I/A/W the manufacturers spec that are FAA approved |  |  |  |  |  |  |  |  |