| 6/27/13 A/C TT 8132.2 N306MH Performed: 50 lube, 100, Hr Airframe inspection. 150, 300, 600, 2000Hr Engine inspections. 1500 hr freewheeling unit insp. Performed 100Hr/ 90 day Corrosion Control inspection. Complied with Engine oil change over to Royco 560 oil 1 month fire Ext inspection. 7 day M/R blade insp Complied with ASB 206L-09-158 T/B fitting Complied with CEB A-73-3075 FCU screws Complied with 1500 hour Bleed Valve insp by installing O/H Bleed Valve Removed bleed Valve P/N 23073353 S/N FF57427, Installed O/H same P/N Bleed Valve S/N FF36235 Complied with 12500 hour Impeller insp by installing serviceable Compressor Removed Engine Assy and Reinstalled same Assy S/N for 1,2,3,4,wheels retire Removed Engine Compressor Assy P/N 23033193 S/N CAC91445 Installed Serviceable Compressor P/N 23033193 S/N CAC91334 Removed Engine Turbine Assy P/N 23035128 S/N CAT98521 Installed O/H Turbine Assy P/N 23035128 S/N CAT97928 1. T/R short shaft coupling re-torque due between 8142.2 – 8157.2 Performed ground run and leak check. Operational check flight required Performed Engine “ check run “ procedures. All work performed in accordance with applicable BHT 206L3 M.M, CR&O, SPM. AC 43.13-2B. This aircraft/engine has been inspected/repaired in accordance with current regulations and is approved for return to service. Details on file at Maritime Helicopters under WO # 2662 CRS# ENRR619D ______________________________________ | 6/27/13 A/C TT 8132.2 N306MH Performed: 50 lube, 100, Hr Airframe inspection. 150, 300, 600, 2000Hr Engine inspections. 1500 hr freewheeling unit insp. Performed 100Hr/ 90 day Corrosion Control inspection. Complied with Engine oil change over to Royco 560 oil 1 month fire Ext inspection. 7 day M/R blade insp Complied with ASB 206L-09-158 T/B fitting Complied with CEB A-73-3075 FCU screws Complied with 1500 hour Bleed Valve insp by installing O/H Bleed Valve Removed bleed Valve P/N 23073353 S/N FF57427, Installed O/H same P/N Bleed Valve S/N FF36235 Complied with 12500 hour Impeller insp by installing serviceable Compressor Removed Engine Assy and Reinstalled same Assy S/N for 1,2,3,4,wheels retire Removed Engine Compressor Assy P/N 23033193 S/N CAC91445 Installed Serviceable Compressor P/N 23033193 S/N CAC91334 Removed Engine Turbine Assy P/N 23035128 S/N CAT98521 Installed O/H Turbine Assy P/N 23035128 S/N CAT97928 1. T/R short shaft coupling re-torque due between 8142.2 – 8157.2 Performed ground run and leak check. Operational check flight required Performed Engine “ check run “ procedures. All work performed in accordance with applicable BHT 206L3 M.M, CR&O, SPM. AC 43.13-2B. This aircraft/engine has been inspected/repaired in accordance with current regulations and is approved for return to service. Details on file at Maritime Helicopters under WO # 2662 CRS# ENRR619D ______________________________________ |
| --- | --- |
| 6/27/13 A/C TT 8132.2 N306MH Performed: 50 lube, 100, Hr Airframe inspection. 150, 300, 600, 2000Hr Engine inspections. 1500 hr freewheeling unit insp. Performed 100Hr/ 90 day Corrosion Control inspection. Complied with Engine oil change over to Royco 560 oil 1 month fire Ext inspection. 7 day M/R blade insp Complied with ASB 206L-09-158 T/B fitting Complied with CEB A-73-3075 FCU screws Complied with 1500 hour Bleed Valve insp by installing O/H Bleed Valve Removed bleed Valve P/N 23073353 S/N FF57427, Installed O/H same P/N Bleed Valve S/N FF36235 Complied with 12500 hour Impeller insp by installing serviceable Compressor Removed Engine Assy and Reinstalled same Assy S/N for 1,2,3,4,wheels retire Removed Engine Compressor Assy P/N 23033193 S/N CAC91445 Installed Serviceable Compressor P/N 23033193 S/N CAC91334 Removed Engine Turbine Assy P/N 23035128 S/N CAT98521 Installed O/H Turbine Assy P/N 23035128 S/N CAT97928 1. T/R short shaft coupling re-torque due between 8142.2 – 8157.2 Performed ground run and leak check. Operational check flight required Performed Engine “ check run “ procedures. All work performed in accordance with applicable BHT 206L3 M.M, CR&O, SPM. AC 43.13-2B. This aircraft/engine has been inspected/repaired in accordance with current regulations and is approved for return to service. Details on file at Maritime Helicopters under WO # 2662 CRS# ENRR619D ______________________________________ | 6/27/13 A/C TT 8132.2 N306MH Performed: 50 lube, 100, Hr Airframe inspection. 150, 300, 600, 2000Hr Engine inspections. 1500 hr freewheeling unit insp. Performed 100Hr/ 90 day Corrosion Control inspection. Complied with Engine oil change over to Royco 560 oil 1 month fire Ext inspection. 7 day M/R blade insp Complied with ASB 206L-09-158 T/B fitting Complied with CEB A-73-3075 FCU screws Complied with 1500 hour Bleed Valve insp by installing O/H Bleed Valve Removed bleed Valve P/N 23073353 S/N FF57427, Installed O/H same P/N Bleed Valve S/N FF36235 Complied with 12500 hour Impeller insp by installing serviceable Compressor Removed Engine Assy and Reinstalled same Assy S/N for 1,2,3,4,wheels retire Removed Engine Compressor Assy P/N 23033193 S/N CAC91445 Installed Serviceable Compressor P/N 23033193 S/N CAC91334 Removed Engine Turbine Assy P/N 23035128 S/N CAT98521 Installed O/H Turbine Assy P/N 23035128 S/N CAT97928 1. T/R short shaft coupling re-torque due between 8142.2 – 8157.2 Performed ground run and leak check. Operational check flight required Performed Engine “ check run “ procedures. All work performed in accordance with applicable BHT 206L3 M.M, CR&O, SPM. AC 43.13-2B. This aircraft/engine has been inspected/repaired in accordance with current regulations and is approved for return to service. Details on file at Maritime Helicopters under WO # 2662 CRS# ENRR619D ______________________________________ |