5/7/13  A/C TT 12817.7  N307MH

R&R trans cowling, particle  separator to FOM

Removed leaking rotor brake calipers. Disassembled Rotor brake calipers, installed new o-rings.

Reassembled calipers IAW BHT 206 SI-2044.

Reinstalled rotor brake calipers. IAW BHT 206 SI-2044.

Performed 180 day emergency float checks.

Complied with 36 month float bottle hydro test.

Performed 180 day float electric inflation valve inspection.

This aircraft/engine has been inspected/repaired in accordance with current regulations and is approved for return to service. Details on file at Maritime Helicopters under WO # 2658

CRS# ENRR619D                                               ______________________________________________________