6-29-13 N 308MH WO # 2664 3175.7 A/F TT

Removed Engine due to metal flakes on chip Detectors .

Removed CAE847697 @ 3175.7 TT 5194 CSN.

Installed Rental CAE848035 @ 1656.1 TT 990.2 TSO 3386 CSN.

Fadec version V5202 installed with rental Eng.

Performed 50hr A/F  lube.

Performed 150, 300 hr Eng Insp

Performed 150 hr fadec Insp.

Perfromed 100hr Barrier filter Insp.

Cleaned, Insp, & lubed barrier filter.

Complied with ASB 407-11-95 PT 2 T/B

Complied with 100hr cargo hook Insp.

Operational check flight & Eng. Power check required.

This aircraft/engine has been inspected/repaired in accordance with current regulations and is approved for return to service. Details on file at Maritime Helicopters under WO # 2664.

CRS# ENRR619D                                               ______________________________________________________